# Lab 1 Assignment: Control Volume Analysis

Prepared by: Team 4

Lucas Rodrigues (rodr1053@purdue.edu)

Arnav Basu (basu41@purdue.edu)

Arnav Shah (shah965@purdue.edu)

Matthew Ginart (mginart@purdue.edu)

School of Mechanical Engineering,

Purdue University

585 Purdue Mall

West Lafayette, IN 47907

Date: 2026.09.18

Submitted to: Dr. Weixiao Shang

Objective:

- Design and carry out a test plan to measure velocity and pressure fields around a circular cylinder at various Reynolds numbers.

- Process the collected data to compute the drag force and drag coefficient.

Schematic:

Figure 1: Control Volume used for Momentum Analysis

Assumptions:

- Steady State

- Long cylinder (2D flow)

- Air properties at standard atmospheric conditions

- No body forces in x-direction

- Incompressible Fluid

Basic Equations:

∂∂tCVρdV+CSρVdA=0 			(1)

∂∂tCVuρdV+CSuρVdA=Fs,x+FB,x 		(2)

FD= CD12ρV2A					(3)

Simplify Eq (2):

∂∂tCVuρdV+CSuρVdA=Fs,x+FB,x 		(2)

Starting from Eq (2), we apply our steady state assumption, and the unsteady term is eliminated. We also apply our “no body forces in x-direction” assumption and eliminate FB,x. This gives us the following: CSuρVdA=Fs,x. Since we are assuming that the fluid is incompressible, we can say that density is constant, factoring it out. We are left with the following equation:

ρCSuVdA=Fs,x (4)

Expand the CS term and apply it on each side. Make sure to use the correct velocity vector component on each side of the CV.

CSuρVdA=ABuρVdA+BCuρVdA+CDuρVdA+DAuρVdA

To assist with this expansion, note that dA= ndA. Additionally, flow is assumed to be incompressible (constant ρ) and each face is going to be contributing along its normal axis. For the AB (left) face that is along -i, BC (top) face +j, CD (right) face +i, and DA (bottom) face -j. For the vertical faces dA will become dy and for the horizontal faces it will become dx. Applying these simplifications, we get the following:

ABuρVdA=ρABu(V∙n)dA=-ρ-0.20.2u2dy,

BCuρVdA=ρBCu(V∙n)dA=+ρ-0.50.8uvdx,

CDuρVdA=ρCDu(V∙n)dA=+ρ-0.20.2u2dy,

DAuρVdA=ρDAu(V∙n)dA=-ρ-0.50.8uvdx

All together and using Eq (4) from the previous part we can see that:

CSuρVdA=Fs,x= ρ--0.20.2u2dyAB+-0.50.8uvdxBC+-0.20.2u2dyCD--0.50.8uvdxDA (5)

It is very important to note that, for example, the u acting on face AB is not the same as the one on CD despite how the equation makes it appear. The integrals do not cancel each other out to make Fs,x=0.

What are the surface forces that can be applied on the CV?

The surface forces acting on our control volume in the x-direction come from both pressure acting on the four surfaces of the CV and the reaction (drag) force from the inner control surface around the cylinder. We will let FD be the drag force exerted by the fluid on the cylinder, which is positive in the x-direction. This means that the cylinder exerts -FD onto the fluid within our control volume. Since pressure always acts opposite our outward normal vectors, we will start by simplifying dF from -pdA to -pndA. Additionally, the top and bottom faces of our CV will not have contributing surface forces in the x-direction since their normal vectors are aligned with the established y-axis. This allows us to find our surface force as follows:

Fs,x=-pnxdAAll Faces-FD=-pnxdAAB+-pnxdACD-FD =pdyAB-pdyCD-FD (6)

Solve for FD:

The final expression for FD can be found as follows using Eq (5) and Eq (6):

Fs,x=pdyAB-pdyCD-FD→FD=pdyAB-pdyCD-FS,x→

FD=pdyAB-pdyCD+ ρ-0.20.2u2dyAB--0.50.8uvdxBC--0.20.2u2dyCD+-0.50.8uvdxDA (7)

Solve for CD:

FD= CD12ρV2A					(3)

Using the expression for FD from Eq (7), we can rearrange Eq (3) and come to a final expression for CD. Because flow is assumed to be 2D, we can evaluate FD and A per unit depth into the page. This allows us to set A=d (the cylinder’s diameter) in this expression. The work is as follows:

FD= CD12ρV2A→CD=FD12ρV2A→

CD=FD12ρV2d

short form

CD=pdyAB-pdyCD+ ρ-0.20.2u2dyAB--0.50.8uvdxBC--0.20.2u2dyCD+-0.50.8uvdxDA 12ρV2d  (8)

(full form)

Note post-processing:

The drag coefficients computed with our gathered data do not follow the expected monotonic trend for a circular cylinder. This is likely due to using too few probes on our BC and DA faces along with a control volume whose height (±2d) is too short to fully contain the wake. This allows for some momentum flux to be unaccounted for as it crosses the top and bottom faces. The measured inlet velocity at Re=5 also differs from the expected value by ~10×, suggesting that there is an error in that testing condition.